# UNG8071_EVB NAND Flash Programming

# 1. Introduction

This document describes how to program the UNG8071_EVB on board NAND flash. The flash binaries and scripts UNG8071_EVB_Demo_Images_Vxx.xx.xx.tar.gz required for the flash programming needs to be downloaded from Microchip.com. The xx.xx.xx is the version of the switch software.

The SAM-BA GUI tool provides a means of programming various Atmel ARM processor-based microcontrollers.

# 2. Evaluation board

Note: The NAND disable jumper may be in different location on your board depending on the rework

# 3. Linux Users

The procedure below is tested with the Ubuntu 14.04.05 LTS distribution.

## 3.1. Flash Programming

- Open a terminal window and extract the image and scripts.

$ tar –xzvf UNG8071_EVB_Demo_Images_vxx.xx.xx.tar.gz

$ cd UNG8071_EVB_demo_images/linux

- Connect the micro usb connector of the UNG8071_EVB to the Linux PC.

- Connect the 5V power to UNG8071_EVB board

- Insert the NAND flash disable jumper and hit the reset MCU Reset button (the /dev/ttyACM0 created. You can see this by executing command ‘$ tail –f /var/log/kernel.log’)

- Remove the NAND disable jumper

- If you are using x86 system then run the 'flash_board' script in the terminal window. If you are using x64 system then run execute ‘flash_board_x64’ script.

$ sudo flash_board

Or

$ sudo flash_board_x64

- Once you see "=== DONE. ===" message you hit the MPU Reset button to reboot the UNG8071_EVB board.

# 4. Windows Users

- Open a terminal window and extract the image and scripts. Winzip or 7-zip tool can be used to extract the downloaded UNG8071_EVB_Demo_Images_vxx.xx.xx.tar.gz file.

- Connect the micro usb connector of the UNG8071_EVB to the Windows PC.

- Connect the 5V power to UNG8071_EVB board.

- Insert the NAND flash disable jumper and hit the reset MCU Reset button.

- Now Windows tries to install the driver. The device ‘AT91 USB to Serial Converter ‘should appear under Ports (COM & LPT). If you see ‘unknown device’ or ’Bossa Program port’ uninstall driver and install the driver present in the ‘windows\sam-ba_3.1.4\driver’ directory.

6. Run the below command to program the NAND flash.

# flash_board.bat